The new pcb difference compared with old ones:

AUTO mode:

The AUTO mode can only be started when robot is on the wire.

Start the robot when inside or outside the wire, there is a symbol showing that robot is not on wire, this is for security consideration

[image: image1.png]



Time schedule:

When the robot voltage is less than 26.5V, it will go for auto recharge 2hours before start time.

When the robot voltage is more than 26.5V, it still stand by.

add two function selection:

Touch sensor and alarm sound.

charging station:

The virtual wire connector on station has positive and negative, the left black port is negative, the virtual wire is extended to the front of station, the right red port is positive, the virtual wire is extended to the back of station.

And main part of robot runs outside of the wire.

[image: image2.png]black terminal
)

red terminal
*)





5.rain sensor 

When the rain sensor activated,there is a raining symbol on the display, and robot enter into HOME mode

[image: image3.png]28.8
BEE O HOME

® Charsins! E

1881 MON 09:23




6.Power cut off and recovery, wire cut off and recovery case

When station power off and robot in Time schedule status, robot enters into HOME mode,then if the power recover before start time, robot finish the scheduled time, if the power recover after start time, robot stay in station and next scheduled day still valid.   
when station power off and robot in AUTO mode, it enters into HOME mode.

